Living in the Environment 19 Edition

Chapter 15 Nonrenewable Energy

Core Case Study: Using Hydrofracking to Produce Oil and Natural Gas (1 of 2)

Oil and natural gas trapped between compressed layers of shale rock formations

New technologies allowed access

Horizontal drilling

Hydraulic fracturing (hydrofracking)

Pump water, sand, and chemicals into cracks

Produces hazardous waste slurry

Core Case Study: Using Hydrofracking to Produce Oil and Natural Gas (2 of 2)

15.1 What Types of Energy Resources Do We Use?

90% of the commercial energy used in the world comes from nonrenewable resources

Oil, natural gas, and coal

Energy resources vary greatly in their net energy

Amount of energy available from a resource minus the amount of energy needed to make it available

Where Does the Energy We Use Come From? (1 of 2)

Energy that heats the earth comes from the sun

Commercial energy is sold in the marketplace

Nonrenewable

Oil, coal, natural gas, and nuclear energy

Renewable

Solar, hydropower, biomass, geothermal, and wind

Where Does the Energy We Use Come From? (2 of 2)

Net Energy: It Takes Energy to Get Energy (1 of 3)

Each step in making energy available uses high-quality energy

Example: oil must be found, pumped, transferred to a refinery, converted to gasoline, and delivered to consumers

Net energy yield

Amount of high-quality energy available from a resource minus the high-quality energy needed to make the energy available

Net Energy: It Takes Energy to Get Energy (2 of 3)

Net energy ratio

Also called energy returned on investment

Energy obtained per unit energy used to obtain it

Energy density

Amount of energy available per kilogram of the resource

Net Energy: It Takes Energy to Get Energy (3 of 3)

15.2 What Are the Advantages and Disadvantages of Using Oil?

Conventional crude oil is abundant

Medium net energy yield

Causes air and water pollution

Releases CO2 into the atmosphere

Unconventional heavy oil from oil shale rock and oil sands

Potentially large supplies

Low net energy yield

Higher environmental impact

We Depend Heavily on Oil (1 of 3)

Crude oil (petroleum)

Contains combustible hydrocarbons

Peak production

Time after which production from a well declines

Crude oil cannot be used as it comes out of the ground

Must be refined using high-quality energy

Petrochemicals–byproducts

We Depend Heavily on Oil (2 of 3)

We Depend Heavily on Oil (3 of 3)

Is the World Running Out of Crude Oil? States (1 of 2)

Proven oil reserves–available deposits

12 OPEC countries have 81% of the world’s proven crude oil reserves

These countries play a role in regulating global prices by agreeing to increase or decrease the amount produced

Increasing shortage of cheap oil

Easy-to-reach deposits are quickly being depleted

Case Study: Oil Production and Consumption in the United States (1 of 3)

Case Study: Oil Production and Consumption in the United States States (2 of 3)

U.S. commercial energy sources

82% from fossil fuels

Largest portion comes from crude oil

U.S. oil consumption exceeds domestic production

Must import oil

Recent rise in domestic production of tight oil from shale rock

Likely to peak around 2020 and then decline

Case Study: Oil Production and Consumption in the United States (3 of 3)

Use of Heavy Oil Has a High Environmental Impact (1 of 5)

Shale oil

Oil that is integrated within bodies of shale rock

As opposed to being trapped between layers of rock

Production involves mining, crushing, and heating the rock

Extracts kerogen that can be distilled

Use of Heavy Oil Has a High Environmental Impact (2 of 5)

Use of Heavy Oil Has a High Environmental Impact (3 of 5)

Oil sands (tar sands) another source of heavy oil

Contains bitumen

Extensive deposits in Canada

Extraction

Clear-cutting forests and strip-mining the land

Low net energy yield

Requires much water

Emits pollutants

Use of Heavy Oil Has a High Environmental Impact (4 of 5)

Use of Heavy Oil Has a High Environmental Impact (5 of 5)

15.3 What Are the Advantages and Disadvantages of Using Natural Gas?

Natural gas

Mostly methane

Propane, butane, and hydrogen sulfide in smaller amounts

Has a medium net energy yield and a fairly low production cost

Burns cleaner than oil and coal

Extracted through horizontal drilling and fracking

Natural Gas Is a Versatile and Widely Used Fuel (1 of 2 )

Liquefied petroleum gas (LPG)

Stored in pressurized tanks for use in rural areas

Liquefied natural gas (LNG)

Can be transported across oceans

Low net energy yield

The United States currently exports to other nations

Natural Gas Is a Versatile and Widely Used Fuel (2 of 2 )

Science Focus 15.1: Environmental Effects of Natural Gas Production and Fracking in the U.S. (1 of 2)

Fracking has several harmful environmental effects

Requires enormous volumes of water

Produces hazardous wastewater

Earthquakes could release into groundwater

Failure of well-casing cement causes contaminated ground water

Natural gas fracking excluded from EPA regulations in 2005

Science Focus 15.1: Environmental Effects of Natural Gas Production and Fracking in the U.S. (2 of 2)

Can Natural Gas Help to Slow Climate Change?

Emits less CO2 per unit of energy than coal

Low price could slow shift to other renewable energy resources

Methane a much more potent greenhouse gas than CO2

Drilling, production, and distribution of natural gas releases large quantities of methane

15.4 What Are the Advantages and Disadvantages of Coal? (1 of 2)

Conventional coal is plentiful

High net energy yield

Low cost

Using it has a high environmental impact

We can produce gaseous and liquid fuels from coal

Lower net energy yields

Higher environmental impacts than conventional coal

Coal Is a Plentiful but Dirty Fuel (1 of 4)

Coal

Solid fossil fuel formed from remains of land plants

Burned in power plants

Generated 40% of the world’s electricity in 2015

Largest consumers of coal

China, the United States, and India

Coal Is a Plentiful but Dirty Fuel (2 of 4)

Environmental costs of burning coal

Mining coal severely degrades land

Water and air pollution

Soot and CO2

Trace amounts of mercury and radioactive materials

Scrubbers remove some pollutants before they leave smokestacks

Produces coal ash that must be safely stored

Coal Is a Plentiful but Dirty Fuel (3 of 4)

Coal Is a Plentiful but Dirty Fuel ( 4 of 4)

We Are Not Paying the Full Cost of Using Coal

Harmful environmental and health costs

Not included in market price of coal-generated electricity

Ways to implement full-cost pricing

Phase out subsidies and tax breaks

Require stricter air pollution controls

Tax CO2 emissions

Regulate coal ash as a hazardous waste

15.4 What Are the Advantages and Disadvantages of Coal? (2 of 2)

The Future of Coal

U.S. coal use dropped 18% between 2007 and 2013

Increased competition from natural gas, wind, and solar power

Grassroots political opposition

Natural gas should overtake coal as largest electricity source by the 2030s

U.S. coal producers are exporting coal

Use is expanding in India, Africa, and Asia

We Can Convert Coal into Gaseous and Liquid Fuels (1 of 2)

Conversion of solid coal to synfuels

Synthetic natural gas (SNG) by coal gasification

Methanol or synthetic gasoline by coal liquefaction

Producing synfuels requires mining of 50% more coal

Lower net energy and cost more to produce than coal

We Can Convert Coal into Gaseous and Liquid Fuels (2 of 2)

15.5 What Are the Advantages and Disadvantages of Using Nuclear Power?

Nuclear power has a low environmental impact and a very low accident risk

Drawbacks

Low net energy yield

High costs

Fear of accidents

Long-lived radioactive wastes

Role in spread of nuclear weapons technology

How Does a Nuclear Fission Reactor Work? (1 of 4)

Controlled nuclear fission reaction in a reactor

Light-water reactors

Boil water to produce steam to spin a turbine

Fueled by uranium ore mined from the earth’s crust

Enriched uranium packed as pellets in fuel rods and fuel assemblies

Control rods absorb neutrons

How Does a Nuclear Fission Reactor Work? (2 of 4)

Water is the usual coolant

Containment shell around the core for protection

Emergency core cooling system

Typical cost to construct

$9–11 billion

United States, France, and Russia

Leading producers of nuclear power in 2014

How Does a Nuclear Fission Reactor Work? (3 of 4)

How Does a Nuclear Fission Reactor Work? (4 of 4)

What Is the Nuclear Fuel Cycle? (1 of 3)

Mining the uranium

Processing and enriching the uranium to make fuel

Using it in a reactor

Safely storing the radioactive waste

Retiring the worn-out plant

Storing its high- and moderate-level radioactive parts safely

What Is the Nuclear Fuel Cycle? (2 of 3)

What Is the Nuclear Fuel Cycle? (3 of 3)

Dealing with Radioactive Nuclear Wastes (1 of 2)

Rods must be replaced every three to four years

Stored in water-filled pools for several years to cool

Transferred to dry casks

Can be processed to remove plutonium

Reprocessing reduces storage time from 240,000 years to about 10,000 years

Costly and produces weapons material

Dealing with Radioactive Nuclear Wastes (2 of 2)

No permanent, secure repository exists today

Nevada Yucca Mountain site plan abandoned

Retiring nuclear plants

Enormous costs

Can Nuclear Power Slow Climate Change?

Nuclear power plants–no CO2 emission

Nuclear fuel cycle–emits CO2

Cutting global CO2 emissions in half between 2015 and 2040 and meeting energy needs would require building 12,000 nuclear power reactors

According to Mark Lynas, climate change author

Controversy about the Future of Nuclear Power (1 of 3)

Nuclear reactors produced 19% of U.S. energy in 2014

67 new nuclear reactors under construction worldwide in 2015

U.S. government provides subsidies, tax breaks, and insurance for the nuclear industry

Accidents have dampened public confidence in nuclear power

Controversy about the Future of Nuclear Power (2 of 3)

New technologies

Advanced light-water reactors

Built-in safety features

Smaller, modular light water reactors

Not yet built and evaluated

Thorium-based reactors

Less costly and safer

Controversy about the Future of Nuclear Power (3 of 3)

Solutions

Reactors must be built so that a runaway chain reaction is impossible.

The reactor fuel and methods of fuel enrichment and fuel reprocessing must be such that they cannot be used to make nuclear weapons.

Spent fuel and dismantled plants must be easy to dispose of without burdening future generations with harmful radioactive waste.

Taking its entire fuel cycle into account, nuclear power must generate a net energy high enough so that it does not need government subsidies, tax breaks, or loan guarantees to compete in the open marketplace.

Its entire fuel cycle must generate fewer greenhouse gas emissions than other energy alternatives.

Case Study: The Fukushima Daiichi Nuclear Power Plant Accident in Japan

March 11, 2011 accident

Triggered by major offshore earthquake and resulting tsunami

Important effects

Increased fear about nuclear power

Revealed single accident can cost $500 billion

Some countries announced phasing out nuclear power

Spurred Japan to reduce energy use

Is Nuclear Fusion the Answer?

Fusion

Two isotopes fused together to form a heavier nucleus

Releases energy

Technology used by the sun to release energy

Technology is very difficult to develop

No approach has produced more energy than it has used

Big Ideas (1 of 2)

Net energy

Key factor in evaluating the long-term usefulness of any energy resource

Conventional oil, natural gas, and coal

Plentiful with moderate to high net energy yields

Use has a high environmental impact

Big Ideas (2 of 2)

Nuclear power fuel cycle

Low environmental impact

Very low accident risk

Downsides

High costs

Low net energy

Long-lived radioactive wastes

Role in spreading nuclear weapons technology

Tying It All Together: Fracking, Nonrenewable Energy, and Sustainability

Horizontal drilling and fracking have greatly increased oil and natural gas production in the United States

Long-term usefulness of an energy resource depends on net energy

Challenge: find ways to reduce harmful environmental impacts of fossil fuels and nuclear energy

Option: shift to renewable energy resources