# Chapter 6 Evaporation and Transpiration

## Introduction

## 6-1 Evaporation

## 6-2 Estimating evaporation

## 6-3 Evaporation control

## 6-4 Transpiration

## 6-5 Transpiration control

## 6-6 Evapotranspiration

## 6-7 estimating evapotranspiration

# Introduction

## 1. Evaporation: a process that water is transferred to atmosphere

## 2. Transpiration: water transfer to atmosphere through vegetations

## 3. Evapotranspiration: evaporation + transpiration

## fig. 6.1

## 4. Factors affecting ET:

## (1) temperature

## (2) vapor pressure

## (3) wind velocity

## (4) nature of the evaporating surface

## Table 6.1 and fig. 6.2

# 6-1 Evaporation

## 1. Evaporation: net rate of vapor transfer

## 2. Factors:

## (1) solar radiation

## (2) differences in vapor pressure

## (3) temperature

## (4) wind

## (5) atmospheric pressure

## (6) quality of evaporating water

## 3. Evaporation opportunity:

## (1) open bodies of water (lakes): 100%

## (2) highly saturated soil: 100%

## (3) soil with low moisture: 0%

## (4) different surfaces provide diverse degrees of evaporation

## (5) vary widely with time

# 6-2 Estimating evaporation

## 1. Methods

## (1) water budget: common

## (2) energy budget: expensive

## (3) mass transfer techniques: expensive

## (4) evaporation pan:least expensive

## 2. Water budget calculations

## (1) simple procedure, but seldom produces reliable results

## (2) equation 6.2

## (3) for monthly or annual evaporation

## (4) E = P – R + ΔS

## E = P – R

## 3. Energy budget

## (1) errors 10% in summer, 20% in winter

## (2) equation 6.3

## (3) unit: cal/cm2-day transfer to cm3/cm2-day

## (4) incoming and reflected radiation

## 4. Mass transfer techniques

## (1) based on turbulent transfer of vapor

## (2) theoretical and empirical equations

## (3) relation between evaporation and vapor pressure (equation 6.10)

## (4) Meyer and Dunne equations (equation 6.12 and 6.13)

## Example 6.1, 6.2

## 5. Evaporation pans

## (1) Class A pan (fig. 2.3): 4 feet in diameter, 10 inch in deep, mounted 12 in above

## ground

## (2) pan coefficient: 0.70 – 0.75

## (3) Penman equation (equation 6.18)

# 6-3 Evaporation control

## 1. Controlling methods:

## (1) storing water in covered reservoirs

## (2) using of underground storage

## (3) controlling aquatic growths

## (4) reservoirs with minimal surface area

## (5) using chemicals

## (6) conveying in closed conduits rather than open channels

# 6-4 Transpiration

## 1. Stomatal transpiration: water escapes through pores in the leaf system

## 2. Factors affecting transpiration

## (1) solar radiation

## (2) temperature

## (3) wind velocity

## (4) vapor pressure gradients

## (5) character of plant

## (6) plant density

## (7) soil moisture content: wilting point

## 3. Measurements of transpiration

## (1) laboratory: using tank and weighing

## (2) field: lysimeter: large containers that water budget is maintained (Table 6.2)

## (3) equation:

## a. Blaney-Criddle (equation 6.22, 6.24)

## b. Penman

## Example 6.3, 6.4

# 6-5 Transpiration control

## 1. Controlling methods

## (1) using chemicals to inhibit water loss

## (2) harvesting of plants

## (3) improved irrigation practices

## (4) remove certain vegetative types

# 6-6 Evapotranspiration

## 1. Determining ET approaches

## (1) theoretical: based on physics of process

## (2) analytical: based on energy or water budget

## (3) empirical:

## 2. Thornthwaite equation (equation 6.25)

## 3. Potential evapotranspiration: the water loss which will occur if at no time there is a

## deficiency of water in the soil for the use of vegetation

## 4. Agricultural Research Service (ARS) equation: (equation 6.26)

# 6-7 estimating evapotranspiration

## 1. Penman method: equation 6.27

## Example 6.5

## 2. Simulating evapotranspiration