# Chapter 9 Hydrographs

## 9-1 Hydrograph components

## 9-2 Hydrograph time relationships

## 9-3 Unit hydrographs

## 9-4 Synthetic unit hydrographs

## 9-5 Hydrograph routing

# 9-1 Hydrograph components

## 1. Four component elements: (fig. 8.3)

## (1) direct surface runoff

## (2) interflow

## (3) groundwater or base flow

## (4) channel precipitation

## direct runoff = (1)+(2)+(4)

## 2. Hydrograph stages: (fig. 9.1)

## (1) concentration curve: rising curve

## (2) crest segment: peak flow

## (3) recession: falling portion

## 3. Total runoff hydrograph: TRH

## sum of base flow plus direct runoff

## 4. Direct runoff hydrograph: DRH

## Equilibrium discharge hydrograph (fig. 9.2)

## 5. Distribution of uniform storm rainfall: depression, detentions, channel (fig. 9.3)

## 6. Hydrograph with basin (fig. 9.4)

## 7. Base flow recession: separate direct surface runoff and base flow components

## (equation 9.1 and 9.2)

## cumulative storage of base flow

## (equation 9.3, Fig. 9.5)

## 8. Graphical method for determining base flow recession constant K (fig. 9.6)

## 9. Base flow separation method (fig. 9.7)

## (1) horizontal line (A to B)

## (2) A to C to D (equation 9.4)

## (3) A to C to E to F

## (4) A to F

## (5) A to C to B

# 9-2 Hydrograph time relationships

## 1. Travel time: time required for direct runoff originating at some point in the channel

## to reach the outlet.

## 2. Wave travel time: surface wave, is faster than the average flow velocity and varies

## with channel shape and other factors.

## 3. Excess-rainfall release time: time required for the last, most remote drop of excess

## rain that fell on the watershed to pass the outlet.

## 4. Time base: time from which the direct runoff begins until the direct-runoff

## component reaches zero. (fig. 9.8)

## 5. Lag time: time form the center of mass of effective rainfall to the center of mass of

## direct runoff. (fig. 9.8)

## 6. Time of concentration = excess rain release time (fig. 9.8, Table 9.1)

## 7. Sheet flow travel time: ASCE kinematic wave equation (equation 9.6)

## 8. Manning’s roughness coefficient (Table 9.2)

## 9. Basin lag time: (equation 9.8)

## (1) time interval form the maximum rainfall rate to the peak rate of runoff

## (2) time from the center of mass of actual rainfall to the peak rate of runoff

# 9-3 Unit hydrographs

## 1. Definitions: hydrograph that one-day rainfall produces a 1 inch depth of runoff over

## the given drainage area

## 2. X-hour unit hydrograph (fig. 9.9)

## 3. Unit hydrograph procedures:

## Example 9.1

## 4. UH applications by lagging methods

## Example 9.2 and Example 9.3

## 5. S-hydrograph method

## Example 9.4

## 6. Instantaneous unit hydrograph (IUH)

## Example 9.5

# 9-4 Synthetic unit hydrographs

## 1. Definition: few data to estimate hydrograph

## 2. Gamma distribution:

## Example 9.6

## 3. Snyder’s method:

## (1) time to peak: equation 9.24

## (2) time base: equation 9.25

## (3) duration: equation 9.26

## (4) peak discharge: equation 9.28

## (5) hydrograph construction

## 4. Dimensionless SCS unit hydrograph

## (1) time to peak: equation 9.31

## (2) peak flow: equation 9.32

## Example 9.7

## 5. Espey 10-minute synthetic unit hydrograph

## urban watershed analysis for time to peak, peak flow, time base, and width at 50 and 75% of the peak flow (equation 9.39 to 9.43)

## 6. Clark’s time-area IUH method

# 9-5 Hydrograph routing

## 1. Definition: predict the temporal and spatial variations of a hydrograph as it traverses

## a river reach or reservoir.

## 2. Applications:

## (1) flood prediction

## (2) evaluations of flood control measures

## (3) assessments of urbanization

## 3. Hydrologic river routing: equation 9.50

## (1) Muskingum method: (fig. 9.21)

## outflow (equation 9.55)

## Example 9.9

## (2) Muskingum-Cunge method:

## (3) SCS Att-Kin TR-20 method:

## 4. Hydrologic reservoir routing: equation 9.79