Appendix C Measurements of stream sediment

Introduction

Ⅰ Suspended sediment

Ⅱ Bedload

Introduction

- Stream sediment: fragmental material and particles that are retained and transported in stream channels

- Erosion: raindrop splash, interrill, rill, gullies, channel, and mass movement

- Sediment information:

  - Evaluation of land use management

  - Design of hydraulic structures

  - Water-quality management

  - Water utilization

  - Aquatic-life management

- Sediments: washload, suspended load, bedload

Ⅰ Suspended sediment

- Sediment variation: in depth, velocity, turbulence, particle size and shape, channel geometry, physical and chemical properties of water, watershed size and topographic characteristics, climatic conditions, and time.

- Sampling

  - Site selection: consideration of data needs and the nature of the flow and channel conditions.

  - Number of verticals: depend on the type of information needed, the degree of accuracy required, and the physical aspects of the river

  - Sampling frequency: higher during floods, lower for snow melt

  - Types of samples:

    - Depth-integrated samples: from surface to about 10 cm above streambed

    - Point-integrated samples: at a specific depth in the stream, large stream

    - Grab samples: small headwater stream, using open container

    - Discrete samples: collect on a specific time during a storm runoff

    - Composite samples: combination of a few discrete samples

  - Procedures for depth-integrating samplers

    - The EDI method: equal discharge increment method (Table C.1, fig. C.1)

    - The ETR method: equal transit rate method, uniform speed to transit

  - Procedures for point-integrating samplers: one-, two-, three-, and five-point methods. 0.6d

- Measuring devices

  - Hand-operated samplers

    - Depth-integrating sampler: US DH-48 (fig. C.2)

    - Point-integrating sampler: US P-61 (fig. C.3)

  - Automatic samplers

    - Mechanical samplers

      - Coshocton-wheel runoff sampler: (fig. C.4, Table C.1)

      - Single, rising-stage sampler: siphon principle, remote sites

    - Electrical pumping samplers: 0.5d (fig. C.5)

    - Optical turbidity systems

      - Jackson turbidity units (JTU)

      - Nephelometric turbidity units (NTU)

    - Acoustic Doppler current profiler: sound by water currents

    - Radioactive turbiometric gages:

- Calculations

  - Suspended-sediment load for a cross section: equation C.2, C.3

  - Mean suspended-sediment concentration: equation C.4

  - Suspended-sediment load for a runoff event: equation C.5

Ⅱ Bedload

- Bedload variation: depends on stream discharge (fig. C.6)

- Sampling

  - Sand-bed rivers: Dutch Arnhem sampler (fig. C.7)

  - Coarse sand and coarse gravel: Helley-Smith sampler (fig. C.8) and VUV sampler (fig. C.7)

- Measuring devices

  - Bedload samplers

    - Basket-type samplers

    - Tray samplers

    - Pressure-difference samplers: fig. C.8

    - Sediment traps

  - Tracers

  - Acoustic and ultrasonic measurement

- Calculations: equation C.6, C.7