Chapter 6 Water resource problems

Introduction

6-1 Water demand and supply

6-2 Water quantity

6-3 Water quality

6-4 Water rights

Introduction

- Water resources problems: water quantity, quality, and timing.

- Too much water (flooding), too little water (drought), water pollution

6-1 Water demand and supply

- Water demand

  - The world:

    - Total freshwater withdrawals increased about 7.14 times in the 90-year period (Table 6.1)

    - Due to population growth, agricultural expansion, industrial development, and were most rapid after World War II

    - Water consumptions of the world (fig. 6.1)

  - The US

    - A per capita withdrawal about 2.87 times greater than the world average (Table 6.2)

    - Agricultural irrigation and industry are the two most important categories in water demand.

  - Projections for the US

    - Factors: population growth, per capita income, energy consumption, agricultural expansion, economic development, and advances in technology

    - A short-term projection is more reliable than a long-term one

    - Projection of total freshwater withdrawals (Table 6.3)

- Water supply

  - The world

    - Total land runoff of the world is 266 mm/year, renewable water.

    - Not all of this water can be utilized due to geographical location or lack of suitable sites for capture.

    - Water supply is an issue of global quantity and distribution.

    - Distribution of arid region climates (fig. 6.3)

    - Water stress index: a measure to assess pressures on water supplies, an index of 1000 m3/person/year is a benchmark.

  - The US

    - Water withdrawals: 80% from streams and reservoirs, 20% from groundwater

    - A large region of US has experienced water shortages every year due to uneven distribution of population, industry, agriculture, and available water. (Table 6.4)

    - The precipitation regime is affected by:

      - position relative to air-mass movement and moisture content of the air

      - proximity to major air pressure systems

      - elevation and other topographic characteristics

    - Seasonal precipitation distribution (fig. 6.4)

6-2 Water quantity

- Drought: water demand is greater than supply, water shortage occurs. If the shortage extends over a significant period of time, it is a drought.

  - Rainmaking

    - Artificially induced rain or snow.

    - The usefulness of rainmaking is limited by a lack of control over the volume, intensity, and distribution of resulting precipitation.

  - Water translocation

    - Distribution of freshwater supplies in nature is very uneven geographically.

    - North Africa, North American, California’s aqueduct system

    - Challenges for new water translocation project:

      - political and environmental issues will create lengthy delays

      - the costs will be extremely high

      - the construction time is too long

      - smaller projects or other approaches with fewer environmental-impact issues will be more attractive to policy-makers

      - environmental groups will insist on greater water use efficiency prior to construction

  - Desalination

    - Techniques: distillation processes or reverse osmosis

    - Energy requirements, costs, and environmental constraints.

  - Water reclamation: water reuse

  - Rain harvesting: rainfall, snowfall, fog-collection system

  - Iceberg harvesting: some problems

    - ownership of icebergs

    - environmental effects on coastal water temperature, fish reproduction, and migration as well as on climatic patterns

    - hazards that might be created if chunks of iceberg were broken off in international shipping lanes

  - Household conservation: using water wisely, increasing the efficiency, and reducing waste

  - Evaporation reduction:

    - Application of mechanical covers to water’s surface

    - Spreading the surface with thin films of chemicals

  - Irrigation efficiency:

    - Better scheduling of water application

    - Use of closed conduits in water conveyance system

    - Using trickle or drip irrigation systems

    - Reducing irrigation water runoff

    - Covering the soil with mulches

  - Vegetation control: water-wasting plants, vegetation management

  - Genetic engineering: salt-resistant and drought-resistant crops

  - Drought forecast

  - Price control

- Floods

  - Forms and causes

    - Flash flood: in mountain streams, small watershed

    - River flood: along major rivers, large watershed, downstream area

    - Urban flood: urbanization increases runoff

    - Coastal flood: tropical storms, typhoons, tsunamis, …

  - Nature of floods

    - Measure for their elevation, area, peak, volume, duration

    - Warm temperature can melt much of snow

    - Hydrological behavior of watershed varies with basin sizes

  - Flood damages: The assessment of flood losses is not an easy job

    - Flood damages increase with time

    - Losses per unit area decrease with increasing watershed size

    - Forested watershed are less damaging

    - Inundation can be a blessing

  - Flood control and management

    - Flood-control measures: reservoirs, channel modification, soil and vegetation treatments, land drainage

    - Flood forecasting: warning system

    - Floodplain zoning: land use pattern (fig. 6.6)

    - Flood insurance: The National Flood Insurance Act of 1968 in US

    - Disaster aids: assistance compensate disaster victims, FEMA

6-3 Water quality

- Water pollutants: all substances that make water unsuitable for intended uses

  - Sediments: physical, chemical, and biological effects

  - Heat: increase chemical reaction, reduce DO, more evaporation,

  - Oxygen-demanding wastes: sewage, manure, industrial wastes, …

  - Plant nutrients:

    - Soil and water erosion

    - Agricultural fertilizers

    - Domestic sewage

    - Livestock wastes, decomposition of plant residuals

    - Phosphate detergents

  - Disease-causing agents: bacteria, viruses, microorganisms

  - Inorganic chemicals and minerals: acid, salts, heavy metals, toxic pollutants

  - Synthetic organic chemicals: Detergents, plastics, oil, pesticides, septic-tank cleaners, phenols, DDT

  - Radioactive substances: nuclear power plants

- Sources of water pollution

  - Point sources vs. non-point sources pollutions

  - Best management practices (BMPs)

  - Ground water can contamination

- Water quality control: scientific research, technological development, resources management, economic analysis, political movements, law enforcement, public participation, education

  - Monitoring and research:

  - Public education: awareness on water-pollution problems

  - Prevention and treatment:

  - Management: water-pollution policies

  - Legislation and enforcement: provide authorities, standards, finance, guidelines, responsibilities, assessment

- Water-quality laws and regulations

  - Clean Water Act of 1977 in US

  - Laws have impacts on forestry and natural resources management:

    - Sections 208: non-point pollution planning

    - Section 303: total maximum daily load, TMDL

    - Section 304: water quality standards

    - Section 319: water pollution assessment and management program

    - Section 404: wetlands and dredging activities

6-4 Water rights

- Diffused water:

  - Water on the ground surface not in connection with a stream channel

  - Regarded as the property of the landowner

- Surface water:

  - Riparian rights: in eastern half of US, lands bordering a watercourse give the landowners special rights to make use of water in the channel

  - Prior appropriation: first-come, first-served, in nine western states in US

  - Hybrid doctrine: in ten other states, riparian and prior appropriation rights

  - Reserved-rights doctrine: US is entitled to claim water rights for instream flows on federal lands or in Indian reservations

- Groundwater: US state may do following

  - Recognize the ownership of groundwater by the overlying landowner

  - Limit the use of groundwater at a reasonable level

  - Provide a special protection for prior users

  - Manage groundwater resources as public property with rights created under a permit program