Chapter 10 Forests and streamflow

Introduction

10-1 Runoff generation

10-2 Watershed discharges

10-3 Forest practices and water quantity

10-4 Forest practices and water quality

10-5 Forest practices and stream habitat

Introduction

- Terms: discharge or runoff is called streamflow

- Water in stream channels comes from:

  - Precipitation intercepted by stream channels

  - Overland flow (surface runoff)

  - Interflow (subsurface runoff)

  - Base flow (groundwater runoff)

10-1 Runoff generation

- Rainfall intensity

  - Effective rainfall: Pe = P – Interception, 70-80%

  - Rainfall intensity: rainfall per unit of time

  - Relation ship for rainfall depth-area-duration and rainfall intensity-duration-return periods (fig. 10.1)

  - Event occurring probability

- Soil infiltration

  - Soil water: soil porosity, mass of water content

  - Infiltration capacity:

    - The Green-Ampt model: equation (10.9) (fig. 10.3)

    - The SCS RCN approach

- Mechanics of runoff

  - Infiltration: gravity and capillary forces cause the infiltrated water to percolate down to deeper layers

  - Rangeland watershed vs. forested watershed

10-2 Watershed discharges

- Hydrograph (fig. 10.4)

  - Prominent features: rising limb, crest, recession limb

  - Direct runoff vs. base flow

- Factors affecting watershed discharge

  - Climate: Rainfall intensity, depth, duration, type, distribution within the watershed, and direction of storm movement impose direct and significant impacts to streamflow regimes and hydrograph shape.

  - Topography: slope, shape, elevation, size, relief ratio, stream density and frequency, and many others (fig. 10.5)

  - Land use:

    - Control watershed vs. treatment watershed (fig. 10.6)

    - Different percentage of forest cover (fig. 10.7)

- Estimation of streamflow

  - Techniques for streamflow estimation

    - Hydrograph analysis to estimate runoff volume and peakflow from rainfall

    - Statistical analysis to estimate runoff from environmental factors or other sites

    - C. Time-series analysis to generate sequential data into the future

    - Hydrologic simulation to generate data based on physical models

  - SCS RCN model (storm runoff volume)

    - Hydrologic soil groups (Table 10.3)

    - Antecedent moisture conditions (AMC) (Table 10.4)

    - RCN for hydrologic soil-cover complexes (Table 10.5)

  - The rational equation (storm peakflow): Q = k CIA

  - The Maning equation (flood flows):

    - Equation Q = (A R0.67 S0.5)/n

    - Manning’s roughness coefficient n (Table 10.7) (Table 10.8)

  - Water balance approach (annual streamflow): for annual or long-term

    - Runoff maps of the nation

    - Empirical equations

    - Water balance approach (Table10.9)

10-3 Forest practices and water quantity

- Water yield

  - Forest-cutting intensity (Table 10.10)

  - Species: transpiration, interception, …(Table 10.11)

  - Precipitation: wet vs. dry

  - Soil topographic conditions: soil type, depth, slope, …

- Timing of streamflow

  - Seasonal distribution of water yield

  - Geographic distribution: east, Rocky Mountain, west

- Flow extremes

  - Peak flows:

    - Cutting forests can cause an increase in peak flows

    - Soil water holding capacity, transpiration

  - Low flows:

  - Flow duration: the percent time of occurrence is greater for flows of lower magnitude than for flows of higher magnitude (fig. 10.9)

- Groundwater

  - Hydraulic conductivity (k): equation 10.35

  - Groundwater in steep terrain: complicated, perched groundwater

  - Groundwater in level terrain: forest lower the groundwater level

10-4 Forest practices and water quality (Table 10.12)

- Water-quality determination

  - Input-output budgets:

    - Chemical-nutrient budgets (Table 10.13)

    - Dilution and leaching effects

    - Water quality is expected to be better for streams in colder regions

  - Water-quality criteria

    - Water use categories for Clean Water Act:

        - Public water supply

        - Recreation and aesthetic value

        - Aquatic life

        - Agriculture

        - Industries

    - EPA’s drinking water standards: (Table 10.14)

- Forest practices

  - Higher water quality in forested watershed due to

    - More uptake of nutrients and ions by plants

    - Fewer runoff and sediment losses

    - Lower rates of organic-matter decomposition and microbial activity

    - Cooler temperature

    - Less management

  - Clearcutting: increase N and P

  - Forest roads: erosion and sedimentation

  - Forest fire:

    - Reduce interception, transpiration, infiltration rates

    - Increase snow accumulation, storm runoff, water yield, sediment, water temperature, and nutrient losses

    - Improve wildlife habitat, species competition, and forest health

  - Mechanical site preparations: (Table 10.15)

- Cumulative watershed effects

  - Cumulative watershed effects: additive, combined, and interactive impacts created by additional activities through time and space

  - Be assessed for multiple activities and land uses on a single watershed

10-5 Forest practices and stream habitat

      - Stream habitat

- Habitat parameters

  - Streamflow: running water affects the transport and deposition of instream particles, streambed structure, plankton populations, and the ability of fish to feed

  - Temperature: changes in stream temperature beyond the optimum level can cause disease outbreaks, retard growth, stop migration, and cause death

  - Dissolved oxygen: required for respiration

  - Sediment: suspended load and bedload (Table 10.16)

  - Substrate (or streambed): most biologically active zone in stream

  - Pools: concave sections along a stream channel where flow velocity is lower, water is deeper, and substrate particles are finer than in adjacent riffles or runs

  - Cover: shading and sheltering effects of cover

- Habitat-rating indexes

  - Advantages:

        - Spatial and temporal comparisons and communication with other streams

        - Constructing habitat maps in poorly sampled areas

        - Evaluating impact scenarios of regulatory alternatives

        - Identifying and prioritizing areas for conservation actions

        - Predicting or assessing impacts of environmental change

  - Environmental parameters for assessing habitat quality

      - Forest practices

- Forestry practices impacts on stream

  - Altering the structure and composition of the riparian vegetation

  - Allowing more solar radiation to reach stram surfaces

  - Generating more sediment to stream channels

  - Decreasing the amount of large woody debris in streams

- On riparian vegetation: removal of riparian vegetation

  - Cutting down supplies of leaves, needles, twigs, and wood to streams

  - Shifting the composition of organic matter inputs

  - Reducing the amount of insects and other animal wastes from canopy

- On water temperature

  - Variations: timber-harvesting affect max. and min. temperature

  - Management: energy received from the sun

  - Impacts:

        - Stimulate primary production due to additional energy

        - Increase decomposition rates of organic matter

        - Decrease dissolved oxygen levels

- On sediment

  - Variations and impacts: increase sediment (Table 10.17)

    - Altering channel morphology

    - Reducing the abundance of food organisms

    - Degrading gravel quality and strata composition

  - Buffer strips:

    - Streamside management zones (SMZs)

    - Buffer strips width from 10 to 100 m

- On large woody debris (LWD)

  - Variations: diameter greater than 10 cm and length greater than 1 m

  - Impacts: morphology-habitat system