Chapter 11 Forests and stream sediment

Introduction

11-1 Soil-erosion processes

11-2 Watershed gross erosion

11-3 Estimation of sediment yield

11-4 Vegetation effects

Introduction

- Stream sediment: transported or deposited through runoff, debris flow, and channel and bank erosion.

- Sediment impairs the physical, chemical, and biological properties of streams and is the greatest water pollutant in stream.

11-1 Soil-erosion processes

- Detachment

  - Acting forces

    - Raindrop (or flow) energy: (Table 11.1)

    - Storm energy: Wischmeier and Smith (1958) equation 11.5

    - Freezing water: water expands as it freezes

    - Soil resistance: erosion, transportation, depositions (fig. 11.1)

  - Detachment rate

    - Interrills: sheet erosion, occurs in areas between rills

    - Rills: occurs in numerous small channels on hillsides by normal tillage

- Transport

  - Rainfall transport: splash action, is a function of rainfall amount and intensity, slope gradient, soil characteristics, wind speed, and microrelief.

  - Runoff transport: overland flow, determined by flow velocity and mass, slope gradient, hydraulic radius, surface roughness, and particle transportability

- Deposition: sediment load exceeds transport capacity

11-2 Watershed gross erosion

- Interrill and rill erosion

  - Empirical: relates observed erosion data to environmental factors through statistical analysis. Equation 11.22 and 11.23

  - Factor-based

    - Universal Soil Loss Equation (USLE): Soil loss A = RKLSCP

      - R: rainfall factor: equation 11.25, 11.26, Table 11.2, fig. 11.2

      - K: soil factor: fig. 11.3

      - L: slope length factor: equation 11.30

      - S: slope steepness factor: equation 11.30

      - C: cover and management factor: Table 11.3, 11.4

      - P: conservation practice factor: Table 11.5

    - The revised universal soil loss equation (RUSLE): in 1987 see p: 250-1

  - Physical

    - Four components: (fig. 11.4)

      - detachment by rainfall

      - detachment by runoff

      - transport capacity by rainfall

      - transport by runoff

    - Models: ANSWERS, CREAMS, EPIC, WEPP,

- Ephemeral gully erosion (fig. 11.5) (Table 11.6)

  - Processes: initiated and carried by channelized flows

  - Estimates:

    - On-site visit procedure: Compound Topographic Index (CTI)

    - Physical model: equation 11.35, 11.36

- Gully erosion

  - Formation

    - Induced by surface runoff: depth and velocity (fig. 11.7)

    - Induced by subsurface runoff: pipes and tunnels formed by macropores (fig. 11.8)

  - Gully types: U-shaped and V-shaped

  - Estimates: equation 11.40

- Mass movements: land creep, landslides, mudflows, debris flows, avalanches, falling rocks

  - Occurrences

    - Factor of safety: equation 11.41, 11.42, 11.43

    - Forest vegetatin tends to maintain more-secure slopes

  - Estimates: GIS and GPS

- Channel erosion

  - Stream-bank erosion: fig. 11.10

  - Sediment transport: Table 11.7

11-3 Estimation of sediment yield

- Sediment delivery ratio (SDR): the ratio between sediment yield in stream or reservoir and the watershed gross erosion is called sediment delivery ratio. Fig. 11.11 and Table 11.8

- Sediment rating-curve: a plot of sediment concentration vs. stream discharge over a wide range of data is called sediment rating-curve (fig. 11.12) (Table 11.9)

- Modified USLE: equation 11.49 for storm erosion estimates

- Statistical models: equation 11.50, 11.51

- Physical models: based on erosion processes and transport mechanisms

11-4 Vegetation effects

- Vegetation mechanisms

  - Mechanical effects: canopy, litter, root (Table 11.10)

  - Hydrologic effects:

    - Interception loss that reduces net precipitation to reach mineral soils

    - Transpiration that depletes soil moisture content

    - Litter and organic matter that slow down flow velocity and prolong times of concentration

- Conservation plants (Table 11.11): selection based on

  - Intended purpose

  - Cause of the erosion problem

  - Site conditions, including soil, topography, and microclimate

  - Plant characteristics

- Forest activity

  - Forest roads

  - Forest cutting and site preparation

  - Forest fires