廖俊瑋著作

- 廖俊瑋、廖學誠 2011 景美溪河川流量變動態勢分析。「2011年中國地理學會學術研討會論文集」，中國地理學會主辦，彰化市，16頁。

- 廖俊瑋、廖學誠 2012太麻里溪流域雨量及流量頻率分析。「2012年台灣地理學會學術研討會論文集」，台灣地理學會主辦，台北市，19頁。

- 廖俊瑋、葉春國、詹仕堅、劉一新、廖學誠 2012 太麻里溪集水區極端水文事件的頻率分析。工程環境會刊，28：89-103。

- 廖學誠、林俊成、葉春國、廖俊瑋、宋健豪 2013氣候變遷下環境敏感區的時空變異及原住民族的因應策略－以台東太麻里溪流域為例(Ⅱ)。國科會專題研究計畫成果報告。36頁。

- 廖俊瑋、廖學誠 2014 景美溪集水區流量及雨量頻率分析。「中國地理學會2014年地理學術研討會」，中國地理學會主辦，台北市。

- 宋健豪、廖俊瑋、廖學誠 2014 蘭陽溪上游集水區降雨量之趨勢分析。「2014森林集水區經營研討會」，林業試驗所，台北市，p:73-92。

- 廖學誠、葉春國、廖俊瑋、宋健豪、陳玉衡 2014 台東太麻里溪流域的地景變遷及社區災害調適之研究(1/2)。國科會專題研究計畫成果報告。78頁。

- 宋健豪、廖俊瑋、廖學誠* 2014 蘭陽溪上游集水區降雨量之趨勢分析。中華林學季刊，47(4)：341-358。

- 廖俊瑋、宋健豪、詹仕堅、廖學誠* 2014石碇雨量站年最大一日、二日、三日降雨量及景美溪下游水文站年最大一日逕流量之頻率分析。中華林學季刊，47(4)：359-372。

- 廖俊瑋、宋健豪、詹仕堅、廖學誠* 2015景美溪集水區年際及豐枯水期水文特性變化之研究。中國地理學會會刊，54：55-72。

太麻里溪集水區極端水文事件的頻率分析

摘 要

頻率分析常被應用於防災規劃、水工設計及災害風險評估中，透過適當的機率分佈可推估最大降雨量或洪峰流量，有助於人們瞭解集水區內的水文特性，提升集水區經營管理之效率。本研究以台東的太麻里溪集水區作為案例，從1982年至2009年間，針對極端水文事件包括年最大日流量以及年一日最大雨量進行頻率分析，比較極端值第一型分佈、對數常態分佈、皮爾森第三型分佈及對數皮爾森第三型分佈之適切性，並據此探討近期氣候變遷之趨勢。結果顯示，研究區內年最大日流量頻率分析以極端值第一型分佈最為適切，後期(1996-2009年)的年最大日流量推估值均比前期(1982-1995年)高出許多，變動率介於37.93%至57.88%間，後期的年最大日流量平均值也比前期高出40.63%，標準差更高出67.94%。另外，年一日最大雨量頻率分析則是以極端值第一型分佈及對數常態分佈為佳，後期的年一日最大雨量推估值均比前期高，變動率介於10.27%至59.25%間，後期的年一日最大雨量平均值比前期高出16.09%，標準差則高出109.30%。整體而言，太麻里溪集水區的年最大日流量及年一日最大雨量均逐漸增高，變動率大，頻度亦增強，顯示出未來的極端水文事件將具有上升之趨勢。

關鍵詞：太麻里溪、頻率分析、極端水文事件、年最大日流量、年一日最大雨量

THE FREQUENCY ANALYSIS OF EXTREME HYDROLOGICAL EVENTS IN THE TAIMALI STREAM WATERSHED

ABSTRACT

The frequency analysis is often applied in disaster prevention planning, hydraulic works design, and hazard risk analysis. It is possible to estimate the maximum volumes of intensive rainfall and peak discharge based on a suitable probability distribution. The frequency analysis is also helpful for people to understand the hydrological characteristics in a watershed, and to enhance the efficiency of watershed management. The Taimali stream watershed is selected as a study area in this research. The extreme hydrological events are analyzed including the annual maximum daily discharge and annual one-day maximum rainfall during the period from 1982 to 2009. There are four probability distributions are compared including extreme-value type I distribution, logarithmic normal distribution, Pearson type III distribution, and logarithmic Pearson type III distribution. Moreover, the current trend of climate change is also discussed based on the frequency analysis. Results show that the extreme-value type I distribution is more suitable for the frequency analysis of annual maximum daily discharge. The estimated annual maximum daily discharges in late stage (1996-2009) are higher than that in early stage (1982-1995). Their variation is from 37.93% to 57.88%. The average of annual maximum daily discharges in late stage is bigger surpassing up to 40.63%, and the standard deviation is also exceeding up to 67.94%. In addition, the extreme-value type I distribution and logarithmic normal distribution are more suitable for the frequency analysis of annual one-day maximum rainfall. The estimated one-day maximum rainfalls in late stage are higher than that in early stage. Their variation is from 10.27% to 59.25%. The average of annual one-day maximum rainfall in late stage is bigger surpassing up to 16.09%, and the standard deviation is also exceeding up to 109.30%. In conclusion, there is an increasing trend for the annual maximum daily discharge and annual one-day maximum rainfall in the Taimali stream watershed. The variation is enhancing and the frequency is strengthening. It reveals that the occurrence probability of extreme hydrological events is rising in the further future.

Keywords: Taimali Stream, Frequency Analysis, Extreme Hydrological Event, Annual Maximum Daily Discharge, Annual One-day Maximum Rainfall